|  | INTERNATIONAL TELECOMMUNICATION UNION |  |  |  | COM 13 – C 547 – E |
| --- | --- | --- | --- | --- | --- |
|  | TELECOMMUNICATION STANDARDIZATION SECTOR STUDY PERIOD 2009-2012 |  |  | April 2010 |  |
|  |  |  |  | English only Original: English |  |
| Question(s): |  | 12/13 |  |  |  |
| STUDY GROUP 13 – CONTRIBUTION 547 |  |  |  |  |  |
| Source: |  | ETRI |  |  |  |
| Title: |  | Proposed modification on functional entities for web service component in Y.NGN-Web |  |  |  |

At the last NGN-GSI meeting on January 2010, we had discussed functional entities for web service component in Y.NGN-Web (Functional Requirements and Architecture of Web Service Component in NGN). In particular, we had a joint session with Q5 at the last meeting. When we reviewed the figure (i.e., Figure 7 in Y.NGN-Web) on the diagram for functional entities for web service component, we had received several comments for this figure at the joint session.

According to discussion results at the last meeting, we need to update the related descriptions and figure for functional entities for further progress. So, this contribution provides the revised figure and relevant descriptions for functional entities for web service component in Y.NGN-Web.

After discussion, we would like to change the current text into the revised text for Section 9.2 in the following page in Y.NGN-Web.

________________

## 9.2 	Functional entities for web service component

Figure 7 shows functional entities for web service control functions and web media functions of web service component in service control functions.

Figure 7.Functional entities for web service component in NGN

Web service component is comprised of the following functional entities:

Web service control functions include:

- PWC-FE (Proxy Web Control - Functional Entity): PWC-FE performs functionalities as proxy between web server and HTTP client through interaction with directory including profile information.

- WSGC-FE (Web Signaling Gateway Control - Functional Entity): WSGC-FE performs functionalities of signalling gateway and session manager.

- PCC-FE (Policy and Charging Control - Functional Entity): PCC-FE instruct PEC-FE and get information of session flow or service usage from PEC-FE at the transport control functions

- WMRC-FE (Web Media Resources Control - Functional Entity): WMRC-FE is in charge of providing controlling resources to the users.

- Web media functions include:

- WMGC-FE (Web Media Gateway Control - Functional Entity): WMGC-FE is controlled by the WSGC-FE in order to allow authorized media flow.

- WMTE-FE (Web Media Transform - Functional Entity): WMT-FE performs media adaptation according to different user devices and transport environments.

(Editor’s Note) Detailed descriptions for each functional entity will be added for further progress. Interactions with transport control functions such as NACF, MMCF, and RACF should be indicated in the figure. The clear distinction of similar functions in service stratum and transport stratum are also required.

________________